УТВЕРЖДЕН

by order of the Ministry

Labor and social security

Russian Federation

от «27» April 2023 г. No. 358н

PROFESSIONAL STANDARD

Specialist in flight operation of unmanned aviation systems (external pilot) in combination with one or more unmanned aircraft with a maximum take-off weight of more than 30 kg

672

Registration number

Content

I. _ General information 	1

II . Описание трудовых функционных в профессиональный стандарт (функциональная карта видео профессиональной электроний) 	3

III . Characteristics of general labor functions 	5

3.1. General labor function " Flight operation of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms" 	5

3.2. General labor function " Flight operation of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms, with the performance of the functions of the crew commander of an unmanned aircraft" 	10

3.3. General labor function " Organization of training crews of unmanned aircraft with a maximum take-off mass of more than 30 kilograms" 	15

3.4. Generalized labor function " Organization of flight activity that implements flight operation of unmanned aviation systems" 	19

IV . Information about organizations - developers of professional standards 	24

# I. _ General information

| Management of unmanned aircraft with a maximum take-off mass of more than 30 kilograms |  | 17.029 |
| --- | --- | --- |
| (name of type of professional activity) |  | Code |

The main purpose of the type of professional activity:

Safe flight operation of unmanned aircraft with a maximum take-off weight of more than 30 kilograms and (or) unmanned aviation systems consisting of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms

Группа занятий:

| 1325 | Managers of transport divisions | 3153 | Airline pilots and specialists in related occupations |
| --- | --- | --- | --- |
| (OKZ code ) | (naming) | (OKZ code) | (naming) |

Отнесние к видам економический программы:

| 51.21 | Activity of cargo air transport |
| --- | --- |
| 52.23.1 | Auxiliary activity related to air transport |
| 52.23.13 | Execution of aviation works |
| (code ОКВЭД ) | (name of type of economic activity) |

# II . Description of job functions included in the professional standard (functional card of the professional activity type)

| Generalized labor functions |  |  | Labor functions |  |  |
| --- | --- | --- | --- | --- | --- |
| Code | Name | Qualification level | Name | Code | уровень (подуровень) qualification |
| A | Flight operation of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms | 3 | Preparation for the flight of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms | A/01. 3 | 3 |
|  |  |  | Flight performance by one or more unmanned aerial vehicles with a maximum take-off weight of more than 30 kilograms | A/02. 3 | 3 |
| B | Flight operation of unmanned aviation systems, including one or more unmanned aircraft with a maximum take-off weight of more than 30 kilograms, with the performance of the functions of the crew commander of the unmanned aircraft | 4 | Organization of preparation for the flight of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms | B/01. 4 | 4 |
|  |  |  | Carrying out the flight of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms with the performance of the functions of the aircraft commander | B/02. 4 | 4 |
|  |  |  | Руководство работы человенные крепевых днего или селично бесполотных уставных удобных с майский злетной массой более 30 клигоров | B/03. 4 | 4 |
| c | Organization of preparation of unmanned aircraft crews with maximum take-off mass of more than 30 kilograms | 6 | Conducting training, flight training and qualification evaluation of specialists in the operation of unmanned aircraft, taking into account the specifics of the work of the aviation organization | C/01. 6 | 6 |
|  |  |  | Quality control of the performance of labor functions by crew members of unmanned aircraft with a maximum take-off mass of more than 30 kilograms, carrying out flight operation | C /02. 6 | 6 |
| d | Организация летной электронный оформация участьующие летную опросузионие unmanned aviation systems organization | 7 | Organization of activities for preparation for flight operation of unmanned aviation systems, including one or several unmanned aircraft | D/01.7 | 7 |
|  |  |  | Organization of control over flight operation in the organization of unmanned aviation systems, including one or several unmanned aircraft | D/02.7 | 7 |
|  |  |  | Functional organization in the organization of flight safety management systems of unmanned aviation systems, including one or several unmanned aircraft | D/03.7 | 7 |

# III. Characteristics of generalized labor functions

## 3.1. Generalized labor function

| Наименование | Flight operation of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms | Code | A | Level of qualification | 3 |
| --- | --- | --- | --- | --- | --- |

| The origin of generalized labor function | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Possible names of occupations, professions | Внешний пилот Specialist in flight operation of unmanned aviation systems |
| --- | --- |

| Requirements for education and training | Secondary education и Training in an educational organization or an organization that trains aviation personnel specialists according to the list of aviation personnel specialists of civil aviation, corresponding to the requirements of federal aviation rules, according to training programs approved by the authorized body in the field of civil aviation |
| --- | --- |
| Требования к опыту практикой работы | - |
| Special conditions for admission to work | Existence of valid external pilot's certificate with corresponding qualification marks 3 Valid medical certificate of the third or first class 4 Отзутствие непогашенной или неснятой комистация за проемение умышленного крепима 4 Absence of administrative punishment for the use of narcotic drugs or psychotropic substances without a doctor's prescription or new potentially dangerous psychoactive substances 4 |
| Other characteristics | Completion of training in accordance with specialist training programs according to the list of specialists of aviation personnel to obtain necessary qualification marks In order to obtain a qualification mark for admission to the execution of aviation works, which is included in the list of aviation works, which provides for the receipt of a document confirming compliance with the requirements of the Federal Aviation Rules of a legal entity, an individual entrepreneur, the employee must undergo training according to the approved program for the execution of aviation works Fulfillment of the functions established by the Federal Rules for the use of air space of the Russian Federation, approved by the Government of the Russian Federation Английский язык at the level established by the employer |

Additional features

| Name of the document | Code | Наименование базасной группи, должнити (профессии) или специальности |
| --- | --- | --- |
| OKZ | 3153 | Airline pilots and specialists in related occupations |
| OKPDTR | 25331 | Operator of ground-based means of controlling unmanned aircraft |

3.1.1. Labor function

| Наименование | Preparation for the flight of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms | Code | A/01. 3 | Qualification level | 3 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Изучение и Анализ аеронавигационной и метерологической информация, касочейсьесься программы программу простоящего фото |
| --- | --- |
|  | Acquaintance with restrictions in the area of flying unmanned aircraft with a maximum take-off weight of more than 30 kilograms along a route (route) using digital flight-information service platforms |
|  | Preparation of the starting-landing platform and deployment of an unmanned aviation system, which includes one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
|  | Forming the flight plan of an unmanned aircraft |
|  | Preparation and submission to air traffic service authorities of submissions for establishing restrictions on the use of airspace and the flight plan of an unmanned aircraft to obtain permission to use airspace |
|  | Conducting pre-flight preparation of an unmanned aviation system, which includes one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms, to be used in accordance with operational documentation and flight instructions |
|  | Preparation and introduction to the on-board navigation complex (autopilot) of pilotless aircraft flight programs |
|  | Making a decision on the execution of a flight together with the commander of an unmanned aircraft taking into account the analysis of the meteorological, aeronautical situation, technical condition of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
| Necessary skills | Receive and analyze information about meteorological, ornithological and aeronautical conditions |
|  | Use special software to compile flight tasks and entering it into the on-board navigation complex (autopilot) of the unmanned aircraft |
|  | Form a flight plan taking into account the features of the functional equipment of the payload, installed on the unmanned aircraft, and the nature of the external cargo being transported |
|  | Оценивать техническое состояние и вечесть к улиции бесполотных авиационных систем |
|  | Form flight and technical documentation |
| Necessary knowledge | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace, including when flying over populated areas, when performing aviation work |
|  | Principles of operation and operation of power plants, systems and equipment |
|  | Operational limitations of unmanned aircraft and their power plants |
|  | Operating data of the flight manual of an unmanned aircraft or an equivalent document |
|  | Influence of the mass and centering of the aircraft on the safety of flights |
|  | Normative legal acts on the establishment of prohibited zones and restricted flight zones, the procedure for obtaining information about prohibited zones and restricted flight zones |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | The procedure for organizing and performing unmanned aircraft flights in segregated airspace |
|  | Radio communication procedure |
|  | Fundamentals of air navigation, aerodynamics and meteorology in the volume necessary for the preparation and flight of unmanned aircraft with a maximum take-off weight of more than 30 kilograms under expected operating conditions |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | The procedure for calculating the flight of an unmanned aircraft and constructing the flight route |
|  | Requirements of federal aviation regulations, regarding the holder of an external pilot's certificate |
|  | The procedure for preparing the flight task and uploading it to the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | Order of preliminary and pre-flight preparation of unmanned aviation system and its elements |
|  | Rules for conducting and preparing flight and technical documentation, including in digital form using specialized services |
| Other characteristics |  |

3.1.2. Labor function

| Наименование | Flight performance by one or more unmanned aerial vehicles with a maximum take-off weight of more than 30 kilograms | Code | A/02. 3 | Qualification level | 3 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Clarification and correction of the flight task in accordance with actual meteorological, ornithological and navigational data |
| --- | --- |
|  | Establishing communication with the body of the United Air Traffic Organization System and obtaining permission to use air space |
|  | Принятие решения на взлет бесполотного возлетного доставка с максимальный злетной массой более 30 килогомов |
|  | Launching an unmanned aircraft with a maximum take-off mass of more than 30 kilograms |
|  | Take-off and remote control (piloting) of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms and (or) control of flight parameters |
|  | Analyzing the aeronavigation, meteorological, ornithological situation during flight assignment |
|  | Execution of the actions established by the Federal Aviation Rules in the event of special cases in the flight of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
|  | Conducting searches in the event of an emergency landing of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
|  | Informing the relevant bodies of the United Air Traffic Management System about deviations from the flight plan or changes in the flight regime when special situations occur in the flight, about the emergency landing by an unmanned aircraft |
|  | Making decisions about the landing of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms, as well as about terminating the flight and returning to the airfield, or about a forced landing in the event of an obvious threat to the environment or the safety of the flight of the unmanned aircraft, in order to save people's lives, prevent damage to the environment environment |
|  | Performing the approach and landing of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms in automatic and (or) manual control mode |
|  | Performing all types of inspections of unmanned aircraft with a maximum take-off weight of more than 30 kilograms in the process of preliminary, pre-flight, inter-flight preparation, preparation for re-flight |
| Necessary skills | Control unmanned aircraft in accordance with the instructions for its flight operation and within the limitations of its characteristics |
|  | Recognize and control threat factors and errors |
|  | Take decisions and control and observe in flight |
|  | Apply knowledge in the field of air navigation |
|  | Operate and service the functional equipment of the payload of the unmanned aircraft, the information transmission and processing system, as well as the external cargo attachment system |
|  | To process the data obtained when using unmanned aircraft |
|  | Осуществлять изображения со служби и оставка и программы air traffic when organizing and executing flights of unmanned aircraft |
|  | Carry out operation of on-board flight data registration systems, information collection and transmission, including photo and video recording systems |
| Necessary knowledge | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace, including when flying over populated areas, when performing aviation work |
|  | Principles of operation and operation of power plants, systems and equipment |
|  | Operational limitations of unmanned aircraft and their power plants |
|  | Operating data of the flight manual of an unmanned aircraft or an equivalent document |
|  | Influence of the mass and centering of the aircraft on the safety of flights |
|  | Normative legal acts on the establishment of prohibited zones and restricted flight zones, the procedure for obtaining information about prohibited zones and restricted flight zones |
|  | The procedure for organizing and performing unmanned aircraft flights in segregated airspace |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | Fundamentals of air navigation, aerodynamics and meteorology in the volume necessary for the preparation and flight of unmanned aircraft with a maximum take-off weight of more than 30 kilograms under expected operating conditions |
|  | Radio communication procedure |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | Requirements of federal aviation rules, regarding the holder of the external pilot's certificate |
|  | The influence of the installation of the system of the functional equipment of the payload and centering on the flight characteristics and on the behavior of remotely piloted aircraft and autonomous aircraft in flight |
|  | Precautions and procedures in emergency situations, including actions taken to avoid dangerous weather conditions, wake turbulence, and other dangerous phenomena for flight |
| Other characteristics |  |

## 3.2. Generalized labor function

| Наименование | Flight operation of unmanned aviation systems, including one or more unmanned aircraft with a maximum take-off weight of more than 30 kilograms, with the performance of the functions of the crew commander of the unmanned aircraft | Code | B | Level of qualification | 4 |
| --- | --- | --- | --- | --- | --- |

| The origin of generalized labor function | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Possible names of occupations, professions | Commander of the unmanned aircraft |
| --- | --- |

| Requirements for education and training | Secondary education 3 и Training in an educational organization or an organization that trains aviation personnel specialists according to the list of aviation personnel specialists of civil aviation, corresponding to the requirements of federal aviation rules, according to training programs approved by the authorized body in the field of civil aviation 4 |
| --- | --- |
| Требования к опыту практикой работы | - |
| Special conditions for admission to work | Having a valid pilot's certificate with relevant qualification marks A valid medical certificate of the third or first class. No unexpired or unexpired criminal record for committing an intentional crime. Absence of administrative punishment for the use of narcotic drugs or psychotropic substances without a doctor's prescription or new potentially dangerous psychoactive substances |
| Other characteristics | Completion of training in accordance with specialist training programs according to the list of specialists of aviation personnel to obtain necessary qualification marks In order to obtain a qualification mark for admission to the execution of aviation works, which is included in the list of aviation works, which provides for the receipt of a document confirming compliance with the requirements of the Federal Aviation Rules, a legal entity, an individual entrepreneur, must undergo training according to an approved program for the execution of aviation works Fulfillment of the functions established by the Federal Rules for the use of air space of the Russian Federation, approved by the Government of the Russian Federation Английский язык at the level established by the employer |

Additional features

| Name of the document | Code | Наименование базасной группи, должнити (профессии) или специальности |
| --- | --- | --- |
| OKZ | 3153 | Airline pilots and specialists in related occupations |
| OKPDTR | 25331 | Operator of ground-based means of controlling unmanned aircraft |
|  | 23456 | Commander (pilot, pilot) of the aircraft - instructor |
| ОКСО | 2.25.02.04 | Flight operation of aircraft |
|  | 2.25.02.05 | Control of air transport |

3.2.1. Labor function

| Наименование | Organization of preparation for the flight of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms | Code | B/01. 4 | Qualification level | 4 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Making the final decision on the execution of the flight based on the analysis of the meteorological, aeronautical situation, the technical condition of the aircraft and the report on the readiness for flight of the crew members of one or several unmanned aircraft |
| --- | --- |
|  | Formation of the flight plan of one or several unmanned aircraft |
|  | Preparation and submission to the air traffic service authorities of submissions for the establishment of restrictions on the use of airspace and the flight plan of one or several unmanned aircraft to obtain permission for the use of airspace |
|  | Checking the actual technical condition of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
|  | Оформление летной и технические документации |
|  | Руководство работы одного или селично вечерние кревей в ходе предлагания бесполотной авиационной система к полету |
|  | Analyzing aeronavigatsionnoi information при подготовке к полету |
| Necessary skills | Receive and analyze information about meteorological, ornithological and aeronautical conditions |
|  | Calculate and issue a flight plan using special software |
|  | Form a flight plan taking into account the features of the functional equipment of the payload, installed on the unmanned aircraft, and the nature of the external cargo being transported |
|  | Оценивать техническое состояние и вечесть к улиции бесполотной авиационной системы |
|  | Form flight and technical documentation |
|  | Formulate tasks and instructions for unmanned aircraft crew members |
| Necessary knowledge | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace, including when flying over populated areas, when performing aviation work |
|  | Principles of operation and operation of power plants, systems and equipment |
|  | Operational limitations of unmanned aircraft and their power plants |
|  | Operating data from the flight manual of an unmanned aircraft or an equivalent document |
|  | Influence of the mass and centering of the aircraft on the safety of flights |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | Normative legal acts on the establishment of prohibited zones and restricted flight zones, the procedure for obtaining information about prohibited zones and restricted flight zones |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | The procedure for organizing and performing unmanned aircraft flights in segregated airspace |
|  | Radio communication procedure |
|  | Fundamentals of air navigation, aerodynamics and meteorology in the volume necessary for the preparation and flight of unmanned aircraft with a maximum take-off weight of more than 30 kilograms under expected operating conditions |
|  | Требования эксплатнационной документации unmanned aviation system |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | The procedure for calculating the flight of an unmanned aircraft and constructing the flight route |
| Other characteristics | - |

3.2.2. Labor function

| Наименование | Carrying out the flight of one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms with the performance of the functions of the aircraft commander | Code | B/02. 4 | Qualification level | 4 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Remote control of take-off and flight (piloting) of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms and (or) control of flight parameters |
| --- | --- |
|  | Ensuring the required flight safety level |
|  | Definition of threat factors and errors for preventing collision with earth |
|  | Execution of the actions established by the Federal Aviation Rules in the event of special cases in the flight of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
|  | Informing the relevant bodies of the United Air Traffic Management System about deviations from the flight plan or changes in the flight regime, when special situations occur in the flight, about making an emergency landing |
|  | Making the final decision about the flight and landing of the unmanned aircraft, as well as the termination of the flight and the return to the airport or the forced landing in case of an obvious threat to the safety of the flight of the unmanned aircraft |
|  | Making decisions about the landing of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms, as well as about terminating the flight and returning to the airfield, or about a forced landing in the event of an obvious threat to the environment or the safety of the flight of the unmanned aircraft, in order to save people's lives, prevent damage to the environment environment |
|  | Performing all types of inspections of unmanned aircraft with a maximum take-off weight of more than 30 kilograms in the process of preliminary, pre-flight, inter-flight preparation, preparation for re-flight |
| Necessary skills | Control unmanned aircraft in accordance with the instructions for its flight operation and within the limitations of its characteristics |
|  | Recognize and control threat factors and errors |
|  | Принимать решения и кафилиатно оформациять контроль и образование в полете |
|  | Apply knowledge in the field of air navigation |
|  | Operate and service the functional equipment of the payload of the unmanned aircraft, the information transmission and processing system, as well as the external cargo attachment system |
|  | Use special software to compile a flight program and enter it into the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | To process the data obtained when using remotely piloted aircraft |
|  | Осуществлять изображение со служби и оставка и программы air traffic when organizing and executing flights of remotely piloted airships |
|  | Осуществлять управление бесполотным высодном тышим чтобы чтобы чтобы программы программы существенное программы или маневров |
|  | Carry out operation of on-board flight data registration systems, information collection and transmission, including photo and video recording systems |
|  | Control the approach and landing of an unmanned aircraft with a maximum take-off weight of more than 30 kilograms in automatic and (or) manual control mode |
|  | Formulate tasks and instructions for unmanned aircraft crew members |
| Necessary knowledge | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace, including when flying over populated areas, when performing aviation work |
|  | Normative legal acts on the establishment of prohibited zones and restricted flight zones, the procedure for obtaining information about prohibited zones and restricted flight zones |
|  | Principles of operation and operation of power plants, systems and equipment |
|  | Operational limitations of unmanned aircraft and their power plants |
|  | Operating data from the flight manual of an unmanned aircraft or an equivalent document |
|  | Influence of the mass and centering of the aircraft on the safety of flights |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | The procedure for organizing and performing unmanned aircraft flights in segregated airspace |
|  | Fundamentals of air navigation, aerodynamics and meteorology in the volume necessary for the preparation and flight of unmanned aircraft with a maximum take-off weight of more than 30 kilograms under expected operating conditions |
|  | Requirements for operational documentation |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | The procedure for calculating the flight of an unmanned aircraft and constructing the flight route |
| Other characteristics | - |

3.2.3. Labor function

| Наименование | Руководство работы человенные крепевых днего или селично бесполотных уставных удобных с майский злетной массой более 30 клигоров | Code | B/03. 4 | Qualification level | 4 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Evaluation of readiness of crew members to perform flight tasks |
| --- | --- |
|  | Organization of pre-flight preparation of the crew of one or several unmanned aircraft |
|  | Проведение разбора полетов |
|  | Performing recording, documenting and accounting in the established order of special cases when operating unmanned aircraft |
|  | Отстранение от переводного дания противного достанию при нализация от переводного достанию противного дания при вечение от проективный пройтинного дания при нализация от переводного противного дания. |
| Necessary skills | Showing signs of fatigue, illness, alcohol or narcotic intoxication in the flight crew members of an unmanned aircraft |
|  | Примениять процедуры процедуния бесплатной комментарии крепов бесполотного доставка от политного задания |
|  | Manage pre-flight preparation of unmanned aircraft crew |
| Necessary knowledge | Regulation of removal of unmanned aircraft crew members from flight |
|  | Руководство по выпечитанию полетов |
|  | Work technologies of unmanned aircraft crew members |
|  | Руководство по управлению безопасностьу полетов |
| Other characteristics | - |

## 3.3. Generalized labor function

| Наименование | Organization of preparation of unmanned aircraft crews with maximum take-off mass of more than 30 kilograms | Code | c | Level of qualification | 6 |
| --- | --- | --- | --- | --- | --- |

| The origin of generalized labor function | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Possible names of occupations, professions | Внешний пилот - инструктор Commander-instructor of unmanned aerial vehicle Руководитель (командир) – instructor of unmanned aviation System |
| --- | --- |

| Requirements for education and training | Secondary professional education - training programs for middle-level specialists on the profile of flight operation of unmanned aviation systems or Secondary professional education - training programs for mid-level specialists in the field of flight operation of aircraft, air traffic control of civil and state aviation and additional professional programs in the field of flight operation of unmanned aircraft systems or Higher education - bachelor's degree and additional professional programs in the field of flight operation of unmanned aviation systems |
| --- | --- |
| Требования к опыту практикой работы | Flying at least 150 hours on airships as an external pilot |
| Special conditions for admission to work | Existence of a valid external pilot's certificate with corresponding qualification marks and "Instructor" qualification mark A valid medical certificate of the third or first class. No unexpired or unexpired criminal record for committing an intentional crime. Absence of administrative punishment for the use of narcotic drugs or psychotropic substances without a doctor's prescription or new potentially dangerous psychoactive substances |
| Other characteristics | Completion of training in accordance with specialist training programs according to the list of specialists of aviation personnel to obtain necessary qualification marks Английский язык at the level established by the employer |

Additional features

| Name of the document | Code | Наименование базасной группи, должнити (профессии) или специальности |
| --- | --- | --- |
| OKZ | 3153 | Airline pilots and specialists in related occupations |
| OKPDTR | 25549 | pilot-instructor |
| ОКСО | 2.25.02.04 | Flight operation of aircraft |
|  | 2.25.02.05 | Control of air transport |
|  | 2.25.03.03 | Air navigation |

3.3.1. Labor function

| Наименование | Conducting training, flight training and qualification evaluation of specialists in the operation of unmanned aircraft, taking into account the specifics of the work of the aviation organization | Code | C/01.6 | Qualification level | 6 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Составление учебно-методического плана занятии, обочники |
| --- | --- |
|  | Training and training of unmanned aircraft crew members with a maximum take-off mass of more than 30 kilograms |
|  | Проведение инструктажей |
|  | Performance of training and training flights |
|  | Отработка с креплами комментарии умений естамить систом, программы и производите в специфических устройство изображение бесполотных хороший содув массой взлетной больше 30 клигомов |
|  | Elimination of deficiencies identified in the work of crews of unmanned aircraft with a maximum take-off mass of more than 30 kilograms |
|  | Conducting classes to improve the qualifications of crew members of unmanned aircraft with a maximum take-off mass of more than 30 kilograms |
|  | Development of training programs on simulators |
| Necessary skills | Control an unmanned aircraft in accordance with the manual for its flight operation and within its operational limitations |
|  | Проверять теоретические дней и практические купиться кревей мебельей бесполотных хайствой содув |
|  | Conduct ground training and training on simulators and unmanned aerial vehicles |
|  | Передавать knowledge and skills in the process of training crew members |
| Necessary knowledge | Methods, methods of training, advanced training and mastery of crew members of unmanned aircraft |
|  | Practical training methods |
|  | Principles of operation and operation of power installations, systems and instrument equipment of unmanned aircraft |
|  | Operational limitations of unmanned aircraft and their power plants |
|  | Operating data from the flight manual of an unmanned aircraft or an equivalent document |
|  | Influence of the mass and centering of the aircraft on the safety of flights |
|  | Fundamentals of aviation psychology |
|  | Human capabilities apply to summer training, including the principles of controlling threat factors and errors |
|  | Security measures related to the imitation of system failures on an unmanned aircraft |
|  | Legislation of the Russian Federation on air transport, regulating flight and training work |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | The procedure for calculating the flight of an unmanned aircraft and constructing the flight route |
|  | Rules and regulations related to the owner of the external pilot's certificate |
|  | The procedure for preparing the flight task and uploading it to the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | Order of preliminary and pre-flight preparation of unmanned aviation system and its elements |
|  | The procedure for organizing and performing unmanned aircraft flights, including in segregated airspace |
|  | Fundamentals of air navigation, aerodynamics and meteorology to the extent necessary for the preparation and execution of unmanned aircraft flights |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | Radio communication procedure |
| Other characteristics | - |

3.3.2. Labor function

| Наименование | Quality control of the performance of labor functions by crew members of unmanned aircraft with a maximum take-off mass of more than 30 kilograms, carrying out flight operation | Code | C/02.6 | Qualification level | 6 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Control, intermediate control of the quality of skills and abilities, knowledge obtained during training on the simulator or in the training flight |
| --- | --- |
|  | Control of independent pilots of foreign pilots, trained persons |
|  | Observing the performance of training and training flights to control the quality of the performance of labor functions by crew members of unmanned aircraft |
|  | Elimination of deficiencies identified in the work of crews of unmanned aircraft with a maximum take-off weight of more than 30 kilograms that carry out flight operations |
|  | Controlling the needs of the organization in the corresponding technical means of training |
|  | Control of compliance with the educational-methodical plan of occupation |
| Necessary skills | Identify deficiencies in the work of crews of unmanned aircraft with a maximum take-off weight of more than 30 kilograms, which carry out flight operations |
|  | Разрабутать контрольно-оценочные местанды для экспрессион и просмотры |
|  | Conduct assessment of skills and abilities, knowledge obtained during training on the simulator or in the training flight |
|  | Расчичивать недвижимость в технических местальных обочниках |
| Necessary knowledge | Methods, methods of training, advanced training and mastery of crew members of unmanned aircraft |
|  | Legislation of the Russian Federation on air transport, regulating flight and training work |
|  | Principles of operation and operation of power plants, systems and equipment |
|  | Operational limitations of unmanned aircraft and their power plants |
|  | Operating data from the flight manual of an unmanned aircraft or an equivalent document |
|  | Influence of the mass and centering of the aircraft on the safety of flights |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | The procedure for calculating the flight of an unmanned aircraft and constructing the flight route |
|  | Rules and regulations related to the owner of the external pilot's certificate |
|  | The procedure for preparing the flight task and uploading it to the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | Order of preliminary and pre-flight preparation of unmanned aviation system and its elements |
|  | The procedure for organizing and performing unmanned aircraft flights, including in segregated airspace |
|  | Fundamentals of air navigation, aerodynamics and meteorology to the extent necessary for the preparation and execution of unmanned aircraft flights |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | Radio communication procedure |
| Other characteristics | - |

## 3.4. Generalized labor function

| Наименование | Организация летной электронный оформация участьующие летную опросузионие unmanned aviation systems organization | Code | d | Level of qualification | 7 |
| --- | --- | --- | --- | --- | --- |

| The origin of generalized labor function | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Possible names of occupations, professions | Flight director of the organization that operates the flight operation of unmanned aviation systems |
| --- | --- |

| Requirements for education and training | Higher education - specialty and additional professional programs in the field of flight operation of unmanned aviation systems, including one or several unmanned aircraft with a maximum take-off weight of more than 30 kilograms |
| --- | --- |
| Требования к опыту практический работы | Not less than three years in aviation transport in positions related to the flight operation of aircraft or unmanned aviation systems |
| Special conditions for admission to work | - |
| Other characteristics | Английский язык at the level established by the employer |

Additional features

| Name of the document | Code | Наименование базасной группи, должнити (профессии) или специальности |
| --- | --- | --- |
| OKZ | 1325 | Managers of transport divisions |
| ОКСО | 2.25.05.04 | Flight operation and application of aviation complexes |
|  | 2.25.05.05 | Operation of aircraft and organization of air traffic |

3.4.1. Labor function

| Наименование | Organization of activities for preparation for flight operation of unmanned aviation systems, including one or several unmanned aircraft | Code | D/01.7 | Qualification level | 7 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Руководство подготовкой к эксполнительных бесполотных авиационных систем с airships |
| --- | --- |
|  | Организация разборов полетов |
|  | Organization of preparation of unmanned aircraft crews for actions in special cases, as well as for actions after forced landing |
|  | Prevention and prevention of acts of illegal interference in the activities of civil aviation in accordance with labor obligations |
|  | Ensuring the observance of a single method of flight performance and summer-methodical training and training of the command-instructor staff |
|  | Ensuring the implementation of the organization's policy in the field of planning, organizing and performing flights, ensuring the safety of flight work in the air space |
|  | Strategic and operational planning of working time of unmanned aircraft crew members |
| Necessary skills | Organize the flight activity of the aviation organization and its structural subdivisions |
|  | Планировать работу по летной эксполнительной бесполотных авиационных систем |
|  | Разрабатывать документации, regulating the flight operation of unmanned aviation systems |
|  | Примениять равила предложения и пресечения октов или негольного вмечения в делайться газиданской авиации |
| Necessary knowledge | Методы огранизационной и правленческой объявление по прогтовка к выплению полетов бесполотных авиационных систем |
|  | Законодательство Российской Федерации, defining the direction of civil aviation development |
|  | Regulatory acts governing summer work |
|  | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace, including when flying over populated areas, when performing aviation work |
|  | Normative legal acts on the establishment of prohibited zones and restricted flight zones, the procedure for obtaining information about prohibited zones and restricted flight zones |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | The procedure for organizing and performing unmanned aircraft flights in segregated airspace |
|  | Radio communication procedure |
|  | Fundamentals of air navigation, aerodynamics and meteorology in the volume necessary for the preparation and flight of unmanned aircraft with a maximum take-off weight of more than 30 kilograms under expected operating conditions |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | The procedure for calculating the flight of an unmanned aircraft and constructing the flight route |
|  | Rules and regulations related to the owner of the external pilot's certificate |
|  | The procedure for preparing the flight task and uploading it to the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | Order of preliminary and pre-flight preparation of unmanned aviation system and its elements |
|  | Rules for maintaining and processing flight and technical documentation, requirements for maintaining and processing flight and technical documentation, including in digital form using specialized services |
| Other characteristics | - |

3.4.2. Labor function

| Наименование | Organization of control over flight operation in the organization of unmanned aviation systems, including one or several unmanned aircraft | Code | D/02.7 | Qualification level | 7 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Control of the implementation of plans for the development of the organization's material-technical and educational base |
| --- | --- |
|  | Control of pre-flight preparation and performance of flights, observance of working regime and rest of flight personnel |
|  | Control of maintaining the qualification of the command-flight and instructor staff |
|  | Quality control and volumes of completed work |
|  | Interaction with controlling and supervisory bodies in the control of flight operation in the organization of unmanned aviation systems |
| Necessary skills | Analyze plans for the development of the organization's material-technical and educational base |
|  | В ыявлять deviations from the organization's material-technical and educational development plans |
|  | В иявлять факти несолдуения регима труда и отдыха летного просонал |
|  | Примениять правильной продавания колфидиция командно-летного и инструкторского состав |
|  | Примениять работа работы с контролирующими и загрузними горнами |
| Necessary knowledge | Технологии работы of aviation organizations that operate unmanned aircraft |
|  | Руководство по управлению безопасностьу полетов |
|  | Методы огранизационной и правленческой объявление по прогтовка к выплению полетов бесполотных авиационных систем |
|  | Законодательство Российской Федерации, defining the direction of civil aviation development |
|  | Regulatory acts governing summer work |
|  | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace , including when flying over populated areas, when performing aviation work |
|  | Normative legal acts on the establishment of prohibited zones and restricted flight zones, the procedure for obtaining information about prohibited zones and restricted flight zones |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | Radio communication procedure |
|  | Fundamentals of air navigation, aerodynamics and meteorology in the volume necessary for the preparation and flight of unmanned aircraft with a maximum take-off weight of more than 30 kilograms under expected operating conditions |
|  | Flight-technical characteristics of the unmanned aviation system and the influence of operational factors on them |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | Rules and regulations related to the owner of the external pilot's certificate |
|  | The procedure for preparing the flight task and uploading it to the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | Rules for maintaining and processing flight and technical documentation, requirements for maintaining and processing flight and technical documentation, including in digital form using specialized services |
| Other characteristics | - |

3.4.3. Labor function

| Наименование | Functional organization in the organization of flight safety management systems of unmanned aviation systems, including one or several unmanned aircraft | Code | D/03.7 | Qualification level | 7 |
| --- | --- | --- | --- | --- | --- |

| Происходень трудовой функции | Original | Kh | Borrowed from the original |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original code | Registration number of the professional standard |

| Labor actions | Organization of investigations of aviation accidents and incidents related to the flight operation of unmanned aviation systems |
| --- | --- |
|  | Разработка of activities aimed at increasing the safety of unmanned aircraft flights in the organization |
|  | Разработка мечарии по предровращению колледиция бесполотных хороший суды ограница с пилотримыми выходними судами |
|  | Carrying out search and rescue work in accordance with official duties in an accident with unmanned airships |
|  | Проведение работ по работы от работых выпационные исшествия, в автомобильно с доставлений образностамия |
| Necessary skills | Analyze information about aviation accidents and incidents related to the flight operation of unmanned aviation systems |
|  | Transform the received information about the investigation of aviation accidents and incidents related to the flight operation of unmanned aircraft systems into a data structure for further development of measures aimed at increasing the safety of unmanned aircraft flights in the organization |
|  | Apply the rules of investigation of aviation accidents and incidents |
|  | Apply search and rescue rules for aviation accidents and incidents |
| Necessary knowledge | Руководство по управлению безопасностьу полетов |
|  | Rules of investigation of aviation accidents and incidents with civilian air vessels |
|  | Search and rescue rules |
|  | Order of preparation and execution of unmanned aircraft flights |
|  | The procedure for operating an unmanned aviation system in the volume of the manual for flight operation |
|  | Order of action in special cases in flight |
|  | Rules and regulations related to the owner of the external pilot's certificate |
|  | The procedure for preparing the flight task and uploading it to the on-board navigation complex (autopilot) of an unmanned aircraft |
|  | Rules for maintaining and processing flight and technical documentation, requirements for maintaining and processing flight and technical documentation, including in digital form using specialized services |
|  | Normative legal acts regulating transport (aviation) security, flight security, radio-technical security |
|  | Rules and procedures established by the air legislation of the Russian Federation for obtaining permission to use airspace, including when flying over populated areas, when performing aviation work |
| Other characteristics |  |

# IV . Information about organizations - developers of professional standards

4.1. Responsible organization-разработчик

Council on professional qualifications of air transport

President Борисенко А. И.

4.2. Наименования organizations-разработчиков

|  | АНО ДПО "Centre of preparation of specialists of unmanned systems", city of Moscow |
| --- | --- |
|  | АО "НПП "Радар ММС", city Saint-Petersburg |
|  | АО "НЦВ Миль и Камов", город Москва |
|  | АО «Почта России», city of Moscow |
|  | Association of partners of the aviation community "Aviation personnel", city Moscow |
|  | Association of employers and enterprises of the industry of unmanned aviation systems "АЕРОНЕКСТ", city of Moscow |
|  | ООО "Aeromax", city Moscow |
|  | ООО «Консалтинговая компания Авиаперсонал», city Moscow |
|  | Russian University of Transport (МИИТ) |
|  | ФГБУ "ВНИИ труда" Минтруда России, город Москва |

Общероссийский классификатор занятий

Общероссийский классификатор видов экономический продукты.

Приказ Минтранса России от 12 сентября 2008 г. № 147 "About the approval of Federal aviation rules "Requirements for aircraft crew members, specialists in technical maintenance of air ships and civil aviation flight controllers" (registered by the Ministry of Justice of Russia on November 20, 2008, registration № 12701) with amendments , by order of the Ministry of Transport of Russia 
on 15 June 2011. № 162 (зарегистрирован Минюстом России 13 июля 2011 г., регистрационный № 21340), 
от 26 декабря 2011 г. № 331 (зарегистрирован Минюстом России 28 марта 2012 г., регистрационный № 23632), 
от 27 декабря 2012 г. № 453 (зарегистрирован Минюстом России 18 февраля 2013 г., регистрационный № 27176), 
от 10 февраля 2014 г. № 32 (зарегистрирован Минюстом России 19 февраля 2014 г., регистрационный № 31362), 
от 16 сентября 2015 г. № 276 (registered by the Ministry of Russia on October 21, 2015, registration number 39387).

Воздушный Code of the Russian Federation.

Decree of the Government of the Russian Federation on March 11, 2010. № 138 "On the regulation of Federal airspace of the Russian Federation", valid until November 1, 2026 .

Общероссийский классификатор профессий рабочий, должнитей сервущих и тарифных разрагов.

Общероссийский классификатор специальностей по образованию.